Final Project Workflow

Edge-Optimized ML Framework for Real-Time Camouflaged Target Detection in Tactical Military Surveillance Systems

# Overview

The project has two distinct phases: an offline training phase that produces a single compressed model file, and a runtime inference phase that consumes that file and runs identically regardless of the video source or target hardware. This separation is deliberate — it is what allows development and testing to happen entirely on a laptop today, with zero behavioral change required when the target hardware (Raspberry Pi 5 + ESP32-CAM) becomes available.

# Phase 1 — Offline Training (dev machine / cloud GPU)

1. Dataset preparation COD10K + CAMO (images) — resize, normalize, augment

↓

2. Lightweight backbone MobileNetV3 / GhostNet — Contribution 1

↓

3. Multi-scale feature fusion Lightweight adaptive BiFPN — Contribution 2

↓

4. DGNet dynamic guidance Foreground guidance + background suppression + fusion

↓

5. Edge-aware attention Coordinate attention + efficient channel attention — Contribution 3

↓

6. Adaptive boundary refinement Edge loss + boundary attention — Contribution 4

↓

7. Segmentation head Reduce to 1 channel, sigmoid, per-pixel probability

↓

8. Total loss + evaluation Seg + edge + fg + bg loss terms ¦ S-measure, Fβw, MAE, E-measure on COD10K / CAMO / NC4K

↓

9. Compression pipeline Pruning + INT8 quantization + ONNX / TFLite / NCNN export — Contribution 5

↓

Output: cod_model_int8.onnx

A single deployable file — identical whether later run on a laptop, an ARM cloud instance, or the Raspberry Pi 5.

# Phase 2 — Runtime Inference (identical code, swappable source)

1. Video source (swappable) Webcam → phone (IP Webcam over WiFi) → ESP32-CAM — same code path throughout

↓

2. Async capture thread + queue Decouples frame acquisition from inference so capture never blocks

↓

3. Motion-adaptive frame skip Skip full inference on static frames; route to propagation instead

↓

4. Tracker-based mask propagation / ONNX Runtime inference Optical flow / lightweight tracker for skipped frames ¦ cod_model_int8.onnx, 4 threads, for full-inference frames

↓

5. Temporal smoothing Blend current output with recent history to remove flicker — Contribution 6

↓

6. Async display / alert Overlay mask on live feed, decoupled from inference thread

↓

Output: live camouflage detection feed

# Validation Progression

Each stage below reuses the exact same runtime code from Phase 2 — only the video_source flag and, where noted, the dataset/footage change. This staged approach is itself worth stating explicitly in the paper's methodology section as evidence of deliberate system design rather than untested hardware assumptions.

| Stage | Source / dataset | Purpose |
| --- | --- | --- |
| 1 | COD10K, CAMO, NC4K (static images) | Core accuracy benchmarking and ablation studies |
| 2 | MoCA-Mask (animal video) | Video/temporal generalization test for the motion-based mechanism (Contribution 6); scope explicitly limited to temporal continuity, not person-specific priors |
| 3 | CPD / COD10K+ (camouflaged people images) | Domain-relevant static evaluation closer to the human/military target |
| 4 | Phone camera via IP Webcam (WiFi stream) | Real network-streaming behaviour; original self-recorded footage of a person in camouflage/patterned clothing against foliage |
| 5 | DVIDS public domain footage | Application-relevant demo/report footage, legally reusable and publishable |
| 6 | ESP32-CAM + Raspberry Pi 5 hotspot | Final hardware-representative deployment and latency validation |

# Six Contributions Summary

| # | Contribution | Addresses |
| --- | --- | --- |
| C1 | Lightweight backbone (MobileNetV3 / GhostNet) | Feature extraction under edge compute constraints |
| C2 | Lightweight adaptive BiFPN | Multi-scale fusion at reduced parameter/FLOP cost |
| C3 | Edge-aware attention (coordinate + ECA) | Spatial and channel attention biased toward object boundaries |
| C4 | Adaptive boundary refinement | Pixel-level mask precision via edge loss and boundary attention |
| C5 | Model compression pipeline | Pruning, INT8 quantization, ONNX/TFLite/NCNN export for CPU-only ARM inference |
| C6 | Temporal continuity module | Frame-skip, mask propagation, and smoothing for real-time video, not just single images |

# Key Corrections Locked Into This Workflow

- TensorRT is NVIDIA-GPU-only and does not run on Raspberry Pi 5; the export target is ONNX Runtime, TFLite, or NCNN instead.

- The ESP32-CAM cannot run any part of the model (240MHz CPU, ~520KB SRAM); it is a capture-only node, all inference happens on the Raspberry Pi 5.

- A Raspberry Pi 5 running its own WiFi hotspot (hostapd + dnsmasq) is preferred over shared router WiFi, since it keeps the whole system infrastructure-independent for field deployment.

- MoCA is animal-only video; it validates the temporal mechanism (C6), not human-camouflage-specific accuracy — this scope is stated explicitly rather than implied.